Parameter Menus

Category

- UPC/EAN

- Code 128

- Code 39

- Code 93

- Interleaved 2 of 5

- Discrete 2 of 5

- Codabar

- MSI/Plessey

- PDF417/MicroPDF 417

- RSS

- Composite Codes

- TLC39

- Postal Codes

- Matrix Codes

- Decode Options

- Data Options

- Scanning Options

- Scan Stand Options

- RS-232 Options

- SSI

- ECI

- Macro PDF

- MCL

- Imaging Options

- Video Options

| Parameter | Parameter Number | Selection | Typical Default |
| --- | --- | --- | --- |
| Set Defaults |  | None | All Defaults |
| UPC/EAN |  |  |  |
| UPC-A | 1 (01h) | Enable (01h) Disable (00h) | Enable |
| UPC-E | 2 (02h) | Enable (01h) Disable (00h) | Enable |
| UPC-E1 | 12 (0Ch) | Enable (01h) Disable (00h) | Disable |
| EAN-8 | 4 (04h) | Enable (01h) Disable (00h) | Enable |
| EAN-13 | 3 (03h) | Enable (01h) Disable (00h) | Enable |
| Bookland EAN | 83 (53h) | Enable (01h) Disable (00h) | Disable |
| Decode UPC/EAN Supplementals | 16 (10h) | Enable (01h) Disable (00h) Autodiscriminate (02h) | Disable |
| Decode UPC/EAN Supplemental Redundancy | 80 (50h) | 2 – 20 (02h – 14h) | 7 |
| Transmit UPC-A Check Digit | 40 (28h) | Enable (01h) Disable (00h) | Enable |
| Transmit UPC-E Check Digit | 41 (29h) | Enable (01h) Disable (00h) | Enable |
| Transmit UPC-E1 Check Digit | 42 (2Ah) | Enable (01h) Disable (00h) | Enable |
| UPC-A Preamble | 34 (22h) | None (00h) System Character (01h) System Character & Country Code (02h) | None |
| UPC-E Preamble | 35 (23h) | None (00h) System Character (01h) System Character & Country Code (02h) | None |
| UPC-E1 Preamble | 36 (24h) | None (00h) System Character (01h) System Character & Country Code (02h) | None |
| Convert UPC-E to A | 37 (25h) | Enable (01h) Disable (00h) | Disable |
| Convert UPC-E1 to A | 38 (26h) | Enable (01h) Disable (00h) | Disable |
| EAN-8 Zero Extend | 39 (27h) | Enable (01h) Disable (00h) | Disable |
| Convert EAN-8 to EAN-13 Type | 224 (E0h) | Type is EAN-13 (01h) Type is EAN-8 (00h) | EAN-13 |
| UPC/EAN Security Level | 77 (4Dh) | 0 – 3 (00h – 03h) | 1 |
| UPC/EAN Coupon Code1 | 85 (55h) | Enable (01h) Disable (00h) | Disable |
| UPC Composite Mode | 344 (F0h 58h) | UPC Never Linked (00h) UPC Always Linked (01h) Autodiscriminate (02h) | UPC Always Linked |
| Code 128 |  |  |  |
| Code 128 | 8 (08h) | Enable (01h) Disable (00h) | Enable |
| UCC/EAN-128 | 14 (0Eh) | Enable (01h) Disable (00h) | Enable |
| ISBT 128 | 84 (54h) | Enable (01h) Disable (00h) | Enable |
| Code 39 |  |  |  |
| Code 39 | 0 (00h) | Enable (01h) Disable (00h) | Enable |
| Trioptic Code 39 | 13 (0Dh) | Enable (01h) Disable (00h) | Disable |
| Code 39 Lengths | 18 (12h) 19 (13h) | Any Length Length Within Range 1 Discrete Length 2 Discrete Lengths (01h – 37h) | Length Within Range: 01-55 |
| Code 39 Check Digit Verification | 48 (30h) | Enable (01h) Disable (00h) | Disable |
| Transmit Code 39 Check Digit | 43 (2Bh) | Enable (01h) Disable (00h) | Disable |
| Code 39 Full ASCII Conversion | 17 (11h) | Enable (01h) Disable (00h) | Disable |
| Convert Code 39 to Code 32 | 86 (56h) | Enable (01h) Disable (00h) | Disable |
| Code 32 Prefix | 231 (E7h) | Enable (01h) Disable (00h) | Disable |
| Code 93 |  |  |  |
| Code 93 | 9 (09h) | Enable (01h) Disable (00h) | Disable |
| Code 93 Lengths | 26 (1Ah) 27 (1Bh) | Any Length Length Within Range 1 Discrete Length 2 Discrete Lengths (04h – 37h) | Length Within Range: 04-55 |
| Interleaved 2 of 5 |  |  |  |
| Interleaved 2 of 5 | 6 (06h) | Enable (01h) Disable (00h) | Disable |
| Interleaved 2 of 5 Lengths | 22 (16h) 23 (17h) | Any Length Length Within Range 1 Discrete Length 2 Discrete Lengths (02h – 36h) | 1 Discrete Length: 14 |
| I 2 of 5 Check Digit Verification | 49 (31h) | None (00h) USS (01h) OPCC (02h) | None |
| Transmit I 2 of 5 Check Digit | 44 (2Ch) | Enable (01h) Disable (00h) | Disable |
| Convert I 2 of 5 to EAN-13 | 82 (52h) | Enable (01h) Disable (00h) | Disable |
| Discrete 2 of 5 |  |  |  |
| Discrete 2 of 5 | 5 (05h) | Enable (01h) Disable (00h) | Disable |
| Discrete 2 of 5 Lengths | 20 (14h) 21 (15h) | Any Length Length Within Range 1 Discrete Length 2 Discrete Lengths (02h – 36h) | 1 Discrete Length: 12 |
| Codabar |  |  |  |
| Codabar | 7 (07h) | Enable (01h) Disable (00h) | Disable |
| Codabar Lengths | 24 (18h) 25 (19h) | Any Length Length Within Range 1 Discrete Length 2 Discrete Lengths (02h – 37h) | Length Within Range: 05-55 |
| CLSI Editing | 54 (36h) | Enable (01h) Disable (00h) | Disable |
| NOTIS Editing | 55 (37h) | Enable (01h) Disable (00h) | Disable |
| MSI Plessey |  |  |  |
| MSI Plessey | 11 (0Bh) | Enable (01h) Disable (00h) | Disable |
| MSI Plessey Lengths | 30 (1Eh) 31 (1Fh) | Any Length Length Within Range 1 Discrete Length 2 Discrete Lengths (01h – 37h) | Any Length |
| MSI Plessey Check Digits | 50 (32h) | One (00h) Two (01h) | One |
| Transmit MSI Plessey Check Digit | 46 (2Eh) | Enable (01h) Disable (00h) | Disable |
| MSI Plessey Check Digit Algorithm | 51 (33h) | Mod 10/Mod 10 (00h) Mod 10/Mod 11 (01h) | Mod 10/Mod 10 |
| PDF417/MicroPDF417 |  |  |  |
| PDF417 | 15 (0Fh) | Enable (01h) Disable (00h) | Enable |
| MicroPDF417 | 227 (E3h) | Enable (01h) Disable (00h) | Disable |
| Decode Linked Symbol | 124 (7Ch) | Ignore (00h) Decode (01h) Autodiscriminate (02h) | Ignore |
| Code 128 Emulation | 123 (7Bh) | Enable (01h) Disable (00h) | Enable |
| RSS |  |  |  |
| RSS-14 | 338 (F0h 52h) | Enable (01h) Disable (00h) | Enable |
| RSS Limited | 339 (F0h 53h) | Enable (01h) Disable (00h) | Enable |
| RSS Expanded | 340 (F0h 54h) | Enable (01h) Disable (00h) | Enable |
| Composite Codes |  |  |  |
| CC-A/B | 342 (F0h 56h) | Enable (01h) Disable (00h) | Enable |
| CC-C | 341 (F0h 55h) | Enable (01h) Disable (00h) | Enable |
| TLC39 |  |  |  |
| TLC39 | 371 (F0h 73h) | Enable (01h) Disable (00h) | Enable |
| Postal Codes |  |  |  |
| US Postnet | 89 (59h) | Enable (01h) Disable (00h) | Enable |
| US Planet | 90 (5Ah) | Enable (01h) Disable (00h) | Enable |
| UK Postal | 91 (5Bh) | Enable (01h) Disable (00h) | Enable |
| Japan Postal | 290 (F0h 22h) | Enable (01h) Disable (00h) | Enable |
| Australian Postal | 291 (F0h 23h) | Enable (01h) Disable (00h) | Enable |
| Matrix Codes |  |  |  |
| Data Matrix | 292 (F0h 24h) | Enable (01h) Disable (00h) | Enable |
| Maxicode | 294 (F0h 26h) | Enable (01h) Disable (00h) | Enable |
| QR Code | 293 (F0h 25h) | Enable (01h) Disable (00h) | Enable |
| Decode Options |  |  |  |
| Linear Code Type Security | 78 (4Eh) | 1- 4 (01h – 04h) | 1 |
| Bi-directional Redundancy | 67 (43h) | Enable (01h) Disable (00h) | Disable |
| Data Options |  |  |  |
| Transmit Code ID Character | 45 (2Dh) | None (00h) AIM Code ID (01h) Symbol Code ID (02h) | None |
| Prefix/Suffix Values Prefix Suffix1 Suffix2 | 99 (63h) 105(69h) 98 (62h) 104(68h) 100 (64h) 106(6Ah) | 4-Digit ASCII Value | ENTER ENTER ENTER |
| Scan Data Transmission Format | 235 (Ebh) | <Data> (00h) <Data> <Suffix1> (01h) <Data> <Suffix2> (02h) <Data> <Suffix1> <Suffix2> (03h) <Prefix> <Data> (04h) <Prefix> <Data> <Suffix1> (05h) <Prefix> <Data> <Suffix2> (06h) <Prefix> <Data> <Suffix1> <Suffix2> (07h) | <Data> |
| Transmit "No Decode" Message | 94 (5Eh) | Enable (01h) Disable (00h) | Disable |
| Transmit LRC Checksum | 126 (7Eh) | Enable (01h) Disable (00h) | Disable |
| Pause Duration | 111 (6Fh) | 0.0 – 9.9 seconds (00h – 63h) | 0.0 seconds |
| Buffer Output Data | 147 (93h) | Enable (01h) Disable (00h) | Disable |
| Scanning Options |  |  |  |
| Decode Attempt Duration (Laser On Time) | 136 (88h) | 0.0 to 9.9 seconds (00h – 63h) | 2 seconds |
| Trigger Mode | 138 (8Ah) | Level (00h) Pulse (02h) Continuous (04h) Blinking (07h) Host (08h) | Level |
| Scanning Mode | 141 (8Dh) | Smart Raster (01h) Always Raster (02h) Programmable Raster (03h) Slab Only (04h) Line Only (05h) Cyclone (06h) Semi-Omni (07h) | Smart Raster |
| Raster Height | 228 (E4h) | 1 – 15 (01h – 0fh) | 15 |
| Raster Expansion Rate | 229 (E5h) | 1 – 15 (01h – 0fh) | 11 |
| Aiming Mode | 382 (F0h 7Eh) | No Aiming Dot (00h) Aiming Dot (01h) Aiming Dot with Extended Timeout (02h) | No Aiming Dot |
| Aim duration | 237 (EDh) | 0.0 to 9.9 seconds (00h – 63h) | 3.0 sec |
| Power Mode | 128 (80h) | Continuous On (00h) Low Power (01h) | Low Power |
| Beep After Good Decode | 56 (38h) | Enable (01h) Disable (00h) | Enable |
| Beeper Tone | 145 (91h) | High (00h) Medium (01h) Low (02h) | High |
| Parameter Scanning | 236 (ECh) | Enable (01h) Disable (00h) | Enable |
| Calibrate Before Each Scan | 369 (F0h 71h) | Enable (01h) Disable (00h) | Enable |
| Calibrate At Powerup | 370 (F0h 72h) | Enable (01h) Disable (00h) | Enable |
| Scan Stand Options |  |  |  |
| Time Delay to Low Power | 146 (92h) | 30 seconds (00h) 1 minute (01h) 2 minutes (02h) 3 minutes (03h) | 30 seconds |
| Timeout Between Same Symbol | 137 (89h) | 0.0 to 9.9 seconds (00h – 63h) | 0.6 seconds |
| Timeout Between Different Symbols | 144 (90h) | 0.0 to 9.9 seconds (00h – 63h) | 0.0 seconds |
| Host Port RS-232 Options |  |  |  |
| Baud Rate | 156 (9Ch) | 300 (01h) 600 (02h) 1200 (03h) 2400 (04h) 4800 (05h) 9600 (06h) 19.2K (07h) 28.8K (09h) 38.4K (08h) 57.6K (0Ah) 115.2K (0Bh) | 9600 |
| Parity | 158 (9Eh) | Odd (00h) Even (01h) Mark (02h) Space (03h) None (04h) | None |
| Check Parity of Received Data | 151 (97h) | Enable (01h) Disable (00h) | Enable |
| Stop Bit Select | 157 (9Dh) | One (01h) Two (02h) | One |
| ASCII Format | 162 (A2h) | 7-Bit ASCII (07h) 8-Bit ASCII (08h) | 8-Bit ASCII |
| Hardware Handshaking | 160 (A0h) | None (00h) Standard RTS/CTS (01h) RTS/CTS Option 1 (02h) RTS/CTS Option 2 (03h) RTS/CTS Option 3 (04h) | None |
| Software Handshaking | 159 (9Fh) | None (00h) ACK/NAK (01h) ACK/NAK with ENQ (02h) ENQ (03h) XON/XOFF (04h) | None |
| Host Serial Response Time-out | 155 (9Bh) | 00 – 9.9 seconds (00h – 63h) | 2.0 seconds |
| Intercharacter Delay | 110 (6Eh) | 00 – 99 ms. (00h – 63h) | 00 |
| Host Serial RTS Line-State | 154 (9Ah) | Low RTS (00h) High RTS (01h) | Low RTS |
| RS-232 Host Type | 163 (A3h) | Standard (37h) ICL (89h) Nixdorf (8Ah) Fujitsu (8Bh) Beetle (8Ch) PDT 3000 (8Dh) | Standard |
| Beep on <BEL> | 150 (96h) | Enable (01h) Disable (00h) | Disable |
| SSC Receive | 187 (BBh) | Enable (01h) Disable (00h) | Enable |
| SSC Transmit | 189 (BDh) | Enable (01h) Disable (00h) | Disable |
| SSI |  |  |  |
| Decode Data Packet Format | 238 (EEh) | Unpacketed (00h) Packeted (01) | Unpacketed |
| Decode Event | 256 (F0h 00h) | Enable (01h) Disable (00h) | Disable |
| Boot Up Event | 258 (F0h 02h) | Enable (01h) Disable (00h) | Disable |
| Parameter Event | 259 (F0h 03h) | Enable (01h) Disable (00h) | Disable |
| Image Capture Notification | 263 (F0h 07h) | Enable (01h) Disable (00h) | Disable |
| Signature Capture Notification | 368 (F0h 70h) | Enable (01h) Disable (00h) | Disable |
| Multipacket Options | 334 (F0h 4Eh) | Option 1 (00h) Option 2 (01h) Option 3 (02h) | Option 1 |
| Interpacket Delay | 335 (F0h 4Fh) | 0 – 255 ms | 0 |
| ECI |  |  |  |
| Delete Character Set ECIs | 230 E6h) | Transmit Character Set ECIs (00h) Delete Character Set ECIs (01h) | Enable |
| ECI Decoder | 232 (E8h) | Enable (01h) Disable (00h) | Enable |
| Macro PDF |  |  |  |
| Macro PDF Transmit/Decode Mode | 188 (BCh) | Buffer All Symbols / Transmit Macro PDF When Complete (00h) Transmit Any Symbol in Set / No Particular Order (01h) Scan in Sequence Only / Transmit in Sequence Without Buffering (02h) Buffer Scans Out of Order / Transmit Scans in Order (03h) | Buffer All Symbols / Transmit Macro PDF When Complete |
| Transmit Each Symbol in Codeword Format | 175 (AFh) | Enable (01h) Disable (00h) | Disable |
| Transmit Unknown Codewords | 186 (BAh) | Enable (01h) Disable (00h) | Disable |
| Escape Character | 233 (E9h) | None (00h) ECI Protocol (01h) GLI Protocol (02h) | None |
| Transmit Macro PDF User – Selected Field |  |  |  |
| Transmit File Name | 176 (B0h) | Enable (01h) Disable (00h) | Disable |
| Transmit Time Stamp | 178 (B2h) | Enable (01h) Disable (00h) | Disable |
| Transmit Block Count | 177 (B1h) | Enable (01h) Disable (00h) | Disable |
| Transmit Sender | 179 (B3h) | Enable (01h) Disable (00h) | Disable |
| Transmit Addressee | 180 (B4h) | Enable (01h) Disable (00h) | Disable |
| Transmit Checksum | 182 (B6h) | Enable (01h) Disable (00h) | Disable |
| Transmit File Size | 181 (B5h) | Enable (01h) Disable (00h) | Disable |
| Transmit Macro PDF Control Header | 184 (B8h) | Enable (01h) Disable (00h) | Disable |
| Last Block Marker | 185 (B9h) | Enable (01h) Disable (00h) | Disable |
| MCL |  |  |  |
| Key Click | 266 (F0h 0Ah) | Enable (01h) Disable (00h) | Enable |
| Sleep Time | 268 (F0h 0Ch) | 5 to 32 seconds (05h – 20h) | 10 seconds |
| Language | 269 (F0h 0Dh) | US English (03h) UK English (02h) French (00h) Dutch (01h) | US English |
| Date Separator | 270 (F0h 0Eh) | Forward Slash (00h) Dash (01h) Colon (02h) None (03h) | Forward Slash |
| Hour Type | 271 (F0h 0Fh) | 12 Hour (01h) 24 Hour (00h) | 12 Hour |
| Decimal Separator | 272 (F0h 10h) | Decimal Point (00h) Comma (01h) | Decimal Point |
| File Separator | 273 (F0h 11h) | Enable (01h) Disable (00h) | Disable |
| Date Format | 275 (F0h 13h) | MM/DD/YY (00h) DD/MM/YY (01h) | MM/DD/YY |
| Imaging Options |  |  |  |
| Decoding Autoexposure | 297 (F0h 29h) | Enable (01h) Disable (00h) | Enable |
| Decoding Illumination | 298 (F0h 2Ah) | Enable (01h) Disable (00h) | Enable |
| Image Capture Autoexposure | 360 (F0h 68h) | Enable (01h) Disable (00h) | Enable |
| Image Capture Illumination | 361 (F0h 69h) | Enable (01h) Disable (00h) | Disable |
| Gain Setting | 311 (F0h 37h) | 128 (0080h) 192 (00C0h) 256 (0100h) 320 (0140h) 384 (0180h) 448 (01C0h) | 192 |
| Exposure Time | 312 (F0h 38h) | 5 ms (004Fh) 10 ms (009Dh) 15 ms (00ECh) 20 ms (013Bh) 25 ms (018Ah) 30 ms (01D8h) | 20 ms |
| Decode Aiming Pattern | 306 (F0h 32h) | Enable (01h) Disable (00h) | Enable |
| Snapshot Mode Timeout | 323 (F0h 43h) | 0-9 (30 seconds – 300 seconds) | 0 (30 seconds) |
| Snapshot Aiming Pattern | 300 (F0h 2Ch) | Enable (01h) Disable (00h) | Enable |
| Image Cropping | 301 (F0h 2Dh) | Enable (01h) Disable (00h) | Disable |
| Crop to Pixel Addresses | 315 (F0h 3Bh) 316 (F0h 3Ch) 317 (F0h 3Dh) 318 (F0h 3Eh) | (0,0) to (639, 479) | 0 top, 0 left, 479 bottom, 639 right |
| Image Resolution | 302 (F0h 2Eh) | Full (00h) 1/2 (01h) 1/3 (02h) 1/4 (03h) | Full |
| JPEG Image Options | 299 (F0h 2Bh) | Quality (01h) Size (00h) | Quality |
| JPEG Quality Value | 305 (F0h 31h) | 5 to 100 | 65 |
| JPEG Size Value | 307 (F0h 33h) | 1 to 150 where value is multiple of 1024 bytes | 40 (41K) |
| Image File Format Selection | 304 (F0h 30h) | BMP (03h) TIFF (04h) JPEG (01h) | JPEG |
| Bits Per Pixel (BPP) | 303 (F0h 2Fh) | 1 (00h) 4 (01h) 8 (02h) | 8 BPP |
| Signature Capture | 93 (5Dh) | Enable (01h) Disable (00h) | Disable |
| Signature Capture File Format | 313 (F0h 39h) | BMP (03h) TIFF (04h) JPEG (01h) | JPEG |
| Signature Capture Bits Per Pixel (BPP) | 314 (F0h 3Ah) | 1 (00h) 4 (01h) 8 (02h) | 8 BPP |
| Signature Capture Width | 366 (F0h 6Eh) | 1 to 640 | 400 |
| Signature Capture Height | 367 (F0h 6Fh) | 1 to 480 | 100 |
| Video Options |  |  |  |
| Video View Finder | 324 (F0h 44h) | Enable (01h) Disable (00h) | Disable |
| Video View Finder Image Size | 329 (F0h 49h) | 800 to 3000 Bytes | 1700 Bytes |
| Target Video Frame Size | 328 (F0h 48h) | 800 to 3000 Bytes | 2200 Bytes |