Elo TouchSystems

A Division of Tyco Electronics Corporation

March 15, 2007

Scanner Test Application Readme

Purpose:

This readme explains how to use your scanner with the provided test applications.

First install the necessary drivers using the SSISDK.exe install file located in the same folder as this readme.  You will also need to install Microsoft .NET Framework Version 2.0 to run the Hall Effect Demo program.

SSIContainer:

One purpose of SSIContainer is to test the scanner’s ability to decode and display a barcode.  The easiest way to test this functionality is to set your scanner to a specific set of parameters.  The following procedure will explain these parameters.

- Scan barcode provided in CD to “Set All Defaults”

- Open SSIContainer and set “Baud” and “Port” settings

- Click on the “Connect” button

- Now you should receive a message in the “Messages” screen that says “Connected”

- In the box labeled “Param Number” type in “238”

- In the box labeled “New Value” type in “1”

- Check the box called “Permanent Param Change”

- Click on “Set Single Parm”

What you have just done is set the scanner to the necessary mode in order to display a barcode on the host computer using SSIContainer.   Now if you scan a barcode it will display the barcode contents in the “Messages” screen of SSIContainer.

Following this example also instructs you on how to make temporary or permanent parameter changes to your scanner.  A list of parameters and their respective options can be found on this CD.

Hall Effect Demo:

One of the main functions of this scanner is that when it is in the cradle, it has the ability to stay in continuous mode.  And when it is removed from the cradle, it will go into a “Trigger Mode,” where the user pushes a trigger in order to activate the scanner.  The Hall Effect Demo program will enable you to test this functionality.  The following procedure explains how to test this feature.

- Scan barcode provided in CD to “Set All Defaults”

- Open up the SSIContainer program and permanently change parameter number “128” to ‘0’ and permanently change parameter number “238” to ‘1.’

- Now open the Hall Effect Demo program and select the “Com Port”

- Click on the “Connect” button

- Now you will receive a message in the “Messages” screen that says the following:

Got Revision info

Software Revision: NBRWDAAF

Board Type: F

Received CAP info

Baud rates supported 300 38400 38400 38400 38400 38400 38400 38400

- Make sure that you get this message only once and that the whole message appears.  If not, please click on “Connect” until you receive this message only once and in its entirety

- Now check the box called “Auto Detect”

- Now your scanner should perform to its specification of being continuously on while in the cradle and only activating with a trigger push while out of the cradle.

- You can test this by following these steps:

  - Put your scanner into the cradle.

    - Now in the Hall Effect Demo program, in the box named “HE Switch Status,” you should see an indicator that says “ON.”  Your scanner should be in the scanning mode.

  - Take your scanner out of the cradle.

    - Now in the Hall Effect Demo program, in the box named “HE Switch Status,” you should see an indicator that says “OFF.”  Your scanner should not be scanning unless you push the trigger button.

If the preceding two steps performed correctly, that means you have successfully tested the scanner’s ability to go from a “Continuous Mode” to a “Trigger Mode.”

==============

Contacting Elo

==============

www.elotouch.com

E-mail: eloinfo@elotouch.com

Elo TouchSystems

A Division of Tyco Electronics Corporation

301 Constitution Drive

Menlo Park, CA 94025

USA

Phone: (800) 557-1458

(650) 361-4700

Fax:     (650) 361-4747

Copyright (©) 2007 Elo TouchSystems

All rights reserved.