# Introduction

ExampleDTRCSharp application is based on C# and can be used with Mono (Has not been tested), VS2005, and VS2005 Express C# versions.

The goal of this application is to provide basic fucntionity using the USB7210 Converter and Symbol Scanner.

# Basic Operation

This screen shot was taken using Vista if you are using XP the Message Screen uses a normal Windows Text.

- Install Driver or plug-in USB7210 into USB port

  - Install driver

- Once the driver is installed select the Comport pull down menu to select the proper serial Port

- Select the connect button  The status bar in the lower right hand conner will display “Attempting Connection”  Once the version information is retrieved from the scanner, the capabilities command is set and also displayed in the Message window.  Then the Status bar will show connected.

- Once connected the Auto Detect check box can be used to automatically detect changes using the HE Switch.  The HE Switch text box will display the information once the Auto Check box is checked.

- Once all operations are complete use disconnect button to end operations.  If the application is shutdown via the close X, it will close the connection automatically.

Some scanners require some additional setup for the HE switch to work properly:

- Once connected use the Parameter Number and new Value fields to enable ACK and Continuous power modes, also the Auto Detect should not be checked.

  - To enable,

  - Type 159 into the Parameter field and 1 into the value and use the Set Single Parameter button.

  - Type 128 into the Parameter field and 0 into the value field and use the Set Single Parameter button.

# Software

There are is a race condition between the GUI and background serial thread.  The GUI operation to the SSIDLL should use a Thread pool to invoke background process for each command.